# Nextion M1 Only Test

## File

[Nextion_M1_Only_Test_Uno.ino](C:/Users/coolg/Documents/Codex/2026-04-19-files-mentioned-by-the-user-acid/Nextion_M1_Only_Test_Uno/Nextion_M1_Only_Test_Uno.ino)

## Purpose

This sketch is for testing only Motor 1 with the existing Nextion screen and RTC clock.

It is meant to isolate lift motion so `M1` can be tuned without `M3`, `M2`, or the full production sequence interfering.

## What It Does

Keeps Nextion communication on pins `10/11`

Keeps DS3231 RTC clock updates on all pages

Supports manual `M1` motion commands from the screen

Supports a simple automatic cycle:

- `M1 DOWN`

- `M1 UP`

## Supported Commands

`START`

Runs one `M1 DOWN -> M1 UP` cycle

`STOP`

Stops motion immediately

`RESET`

Stops motion and clears the test state back to idle

`STATUS`

Logs the current sequence state

`M1F`

Manual Motor 1 forward

`M1R`

Manual Motor 1 reverse

## Pin Use

`M1 PUL -> pin 2`

`M1 DIR -> pin 3`

`M1 ENA -> pin 8`

`Nextion TX/RX -> pins 10/11`

`RTC SDA/SCL -> A4/A5`

## Key Tuning Values

These are the main values to change during bench tuning:

const unsigned long STEP_INTERVAL_M1_US = 800UL;

const unsigned long STEP_01_M1_DOWN_MS  = 8550UL;

const unsigned long STEP_02_M1_UP_MS    = 7850UL;

Meaning:

`STEP_INTERVAL_M1_US`

Lower value = faster stepping

Higher value = slower stepping

`STEP_01_M1_DOWN_MS`

Total Motor 1 down travel time

`STEP_02_M1_UP_MS`

Total Motor 1 up travel time

## Nextion Fields Updated

Main page objects used by this sketch:

`tClock`

`tState`

`tFault`

`tPress`

`tRevs`

`tEstop`

`tDriver`

`tCycle`

`tLsRet`

`tLsExt`

`tEnc`

`tMotor`

`tMotorfwd`

`tMotorrev`

Log page objects:

`tClock1`

`tFrame`

`tLog`

Extra page clock:

`tClock2`

## Display Meaning

`tState`

Current motion state like `M1 DOWN`, `M1 UP`, or `IDLE`

`tFault`

Current sequence step label

`tLsRet`

`M1 DONE`

`tLsExt`

`M1 POS: DOWN/UP`

`tMotorfwd`

`M1 ACTIVE`

`tMotorrev`

Current step interval value

## Notes

`RTC_SET_TIME_ON_BOOT` is currently `false`

This sketch is time-based, not sensor-based

Travel accuracy depends on timing, motor performance, and whether any steps are missed

## Best Use

Use this sketch when the goal is:

tune `M1` direction

tune `M1` speed

tune `M1` down/up travel

verify the Nextion and RTC still work while only `M1` is active