# Nextion M3 Only Test

## File

[Nextion_M3_Only_Test_Uno.ino](C:/Users/coolg/Documents/Codex/2026-04-19-files-mentioned-by-the-user-acid/Nextion_M3_Only_Test_Uno/Nextion_M3_Only_Test_Uno.ino)

## Purpose

This sketch is for testing only Motor 3 with the existing Nextion screen and RTC clock.

It is meant to isolate tilt motion so `M3` speed and travel can be tuned without `M1`, `M2`, or the full production sequence interfering.

## What It Does

Keeps Nextion communication on pins `10/11`

Keeps DS3231 RTC clock updates on all pages

Supports manual `M3` motion commands from the screen

Supports a simple automatic cycle:

- `M3 TILT OUT`

- `M3 TILT HOME`

## Supported Commands

`START`

Runs one `M3 OUT -> M3 HOME` cycle

`STOP`

Stops motion immediately

`RESET`

Stops motion and clears the test state back to idle

`STATUS`

Logs the current sequence state

`M3F`

Manual Motor 3 forward

`M3R`

Manual Motor 3 reverse

## Pin Use

`M3 STEP -> pin 6`

`M3 DIR -> pin 7`

`Shared ENA -> pin 8`

`Nextion TX/RX -> pins 10/11`

`RTC SDA/SCL -> A4/A5`

## Key Tuning Values

These are the main values to change during bench tuning:

const unsigned long STEP_INTERVAL_M3_US      = 1800UL;

const unsigned long STEP_01_M3_TILT_OUT_MS   = 3000UL;

const unsigned long STEP_02_M3_TILT_HOME_MS  = 3000UL;

Meaning:

`STEP_INTERVAL_M3_US`

Lower value = faster stepping

Higher value = slower stepping

`STEP_01_M3_TILT_OUT_MS`

Total tilt-out travel time

`STEP_02_M3_TILT_HOME_MS`

Total tilt-home travel time

## Nextion Fields Updated

Main page objects used by this sketch:

`tClock`

`tState`

`tFault`

`tPress`

`tRevs`

`tEstop`

`tDriver`

`tCycle`

`tLsRet`

`tLsExt`

`tEnc`

`tMotor`

`tMotorfwd`

`tMotorrev`

Log page objects:

`tClock1`

`tFrame`

`tLog`

Extra page clock:

`tClock2`

## Display Meaning

`tState`

Current motion state like `TILT OUT`, `TILT HOME`, or `IDLE`

`tFault`

Current sequence step label

`tLsRet`

`M3 POS: OUT/HOME`

`tEnc`

`M3 DONE`

`tMotor`

`M3 ACTIVE`

`tMotorfwd`

Current step interval value

`tMotorrev`

Current tilt travel time

## Notes

`RTC_SET_TIME_ON_BOOT` is currently `false`

This sketch is time-based, not sensor-based

If you slow `M3` by increasing `STEP_INTERVAL_M3_US`, and still want the same travel, increase the tilt time constants too

## Best Use

Use this sketch when the goal is:

tune `M3` direction

tune `M3` speed

tune `M3` tilt-out and tilt-home travel

verify the Nextion and RTC still work while only `M3` is active